[CEN 323 ] Computer Organization & Assembly Language

[CEN 323 ] Computer Organization & Assembly Language

Lab 5    Journal

Lab 5    Journal

Name : Fatima Asad

Enrollment Number: 01-135232-020

# Lab Deliverables (submission guideline)

- Create a folder named after your enrollment number and add the following files:

  - Create a folder named YourRollNumber_Lab5 containing:

- .asm files of the three practice tasks.

- Add full screen screenshots of the code and output in this word file.

  - Zip the Folder and upload to LMS.

# Lab Task

## Task 1:

Code (TEXT):

org 1000h

.data

streven db 10,13, "the number was even $"

strodd db 10,13, "the number was odd $"

A db ?

.code

mov ah, 1

int 21h

mov ah, 0

mov bl, 2

div bl

cmp ah,0

je even

mov ah, 9

LEA dx,strodd

int 21h

jmp e

even:

mov ah, 9

LEA dx,streven

int 21h

e:

Screenshot (CODE):

Screenshot (OUTPUT):

## Task 2:

Code (TEXT):

.model small

.data

str1 db "Grade = A ",10,13,"$"

str2 db "Grade = B ",10,13,"$"

str3 db "Grade = C ",10,13,"$"

str4 db "Grade = F ",10,13,"$"

str5 db "Marks Are Not valid",10,13,"$"

msg1 db 101

.code

main:

mov ax, @data

mov ds, ax

cmp msg1, 100

JG notValid

cmp msg1, 0

JL notValid

cmp msg1, 90

JGE a

cmp msg1, 80

JGE b

cmp msg1, 50

JGE c

lea dx, str4

mov ah, 09

int 21h

JMP exit

notValid:

lea dx, str5

mov ah, 09

int 21h

JMP exit

a:

lea dx, str1

mov ah, 09

int 21h

JMP exit

b:

lea dx, str2

mov ah, 09

int 21h

JMP exit

c:

lea dx, str3

mov ah, 09

int 21h

exit:

mov ah, 4ch

int 21h

end main

Screenshot (CODE):

Screenshot (OUTPUT):